## 1. Findings

Analysis of the 24-hour telemetry sample (5,000 readings across 10 turbines) identified two units requiring urgent maintenance:

- T-04  - average temperature 90.6°C, exceeding the 85.0°C threshold. Sustained overheating indicates a cooling system fault.

- T-07  - peak vibration 25.0 mm/s, exceeding the 15.0 mm/s threshold. A single spike at this magnitude is sufficient to indicate mechanical failure risk.

The remaining eight turbines are operating within safe limits.

## 2. Architecture Justification

The proposed architecture replaces the single legacy server with a fully managed, event-driven AWS pipeline. The core components are:

- Amazon Kinesis Data Streams - decouples ingestion from processing. The legacy server crashed because both happened on the same machine; Kinesis acts as a durable buffer, absorbing data spikes without data loss even if a downstream component fails.

- AWS Lambda (containerised anomaly script) - the same containerised Python script from Artefact 1 runs as a stream processor, applying the anomaly rules in real time as data arrives.

- Amazon Timestream + SNS - recent readings land in a fast time-series database powering live dashboards, with SNS alerts paging engineers the moment a turbine breaches a threshold.

- Kinesis Firehose + Amazon S3 - all raw readings are archived in a data lake as compressed Parquet files for long-term analysis and audit.

## 3. Cost Optimisation

## The most impactful cost reduction is tiered storage. An S3 lifecycle policy automatically transitions raw telemetry to S3 Glacier after 30 days, reducing storage costs by up to 80% on historical data. Combined with writing cold data as compressed Parquet rather than raw JSON, storage spend on a continuous high-volume pipeline is significantly reduced without losing any data